Section 6. Statement of Requirements

Contents

1. List of Supplies and Related Services	2

2. Delivery and Completion Schedule	4

3. Specification and Compliance Sheet	5

4.Inspections and Tests  	17

1.	List of Supplies and Related Services

Procurement Reference Number: MUK/SPLS/2007-08/00121

- Brief Description of Supplies and Related Services

| Item number | Brief Description of Supplies and Related Services | Quantity | Unit of Measure |
| --- | --- | --- | --- |
| 1 | Supply and installation of video conferencing equipment | 1 | Each |

- Project details

This videoconferencing project involves:

  - Supply and installation of specified equipment and any other equipment that may be needed to offer a complete solution.

  - Supply and installation of a control room within the videoconferencing facility. The main equipment (codec, audio base unit, Cisco switch and UPS) will be mounted in an equipment rack which will be located in this room (as shown in diagram 1).

  - Supply and installation of all power and data cabling and outlets through PVC trunking. Trunking must have a dividing fillet to separate the power cabling from the rest of the cables

  - The power equipment to supplied and installed includes:
     - rack mount UPS (5KVa) to provide power for all the equipment. 
      - complete single phase Main Distribution Board for the videoconferencing facility.
      - distribution cable for the mains power

  - Supply and installation of a Cisco switch and all necessary termination devices to cater for the data outlets. Installation of a fiber (Gigabit multimode) uplink to the server room on ground floor.

  - Supply and installation of special fluorescent lighting fixtures.

  - Supply and installation of acoustic quality curtains for three sides of the room (see diagram    
     1). The curtain boxes for these curtains should be fixed at ceiling level.

2.	Delivery and Completion Schedule

Procurement Reference Number: MUK/SPLS/2007-08/00121

The delivery or completion period shall commence from the date of contract award. Refer to the Incoterm in the BDS 14.6(a)(i) for the interpretation of the delivery period.

| Item number | Brief Description of Supplies or Related Services | Delivery/ Completion Period (days/wks/mths) | Delivery Point/ Site |  |
| --- | --- | --- | --- | --- |
| 1 | Supply and installation of video conferencing equipment | 8 weeks | Makerere University |  |

3.	Specification and Compliance Sheet

Procurement Reference Number: MUK/SPLS/2007-08/00121

Column b states the minimum technical specification of the item(s) required by the Procuring and Disposing Entity.

The Bidder is to complete column c with the technical specification of the item(s) offered and to state “comply” or “not comply” and give details of the areas of non-compliance.

| Item No | Technical Specification Required | Compliance of Specification Offered |
| --- | --- | --- |
| a | b | c |
|  | Detailed statement of work |  |
|  | Videoconferencing codec |  |
|  | Rack mount 2U design |  |
|  | 15U equipment rack (to be located in control room and covering all equipment with locking facility) |  |
|  | Embedded multi-site functionality: 4 video and 3 audio sites, audio and video transcoding, voice activated switching, downspeeding and IPLR |  |
|  | DVD, MPEG, VCR, CD recording/playback |  |
|  | Content sharing |  |
|  | Controller for all devices |  |
|  | User interface: Remote control, System management via embedded web server, SNMP, Telnet, SSH, FTP, Directory services , Remote software upload: via web server, ftp server, 1 x RS-232 local control and diagnostics, On-screen menu system |  |
|  | Cable for connecting all equipment supplied (with appropriate length to cater for equipment placement positions. To be run through trunking where necessary) |  |
|  | Warranty: 2 years (parts and labour) |  |
|  | Video features |  |
|  | Native 16:9 Widescreen |  |
|  | Advanced Screen Layouts |  |
|  | Picture in Picture (PIP) |  |
|  | Picture outside Picture (POP) & Large POP |  |
|  | Side by Side |  |
|  | PC Zoom |  |
|  | Intelligent Video Management |  |
|  | Simultaneous videoconference & local PC mode |  |
|  | Local Auto Layout |  |
|  | Video inputs (5 inputs) |  |
|  | 1 x 9 Pin DSUB:HD Main camera or S-video & control main camera |  |
|  | 1 x S-video: auxiliary/document camera |  |
|  | 1 x RCA/ composite: document camera/aux |  |
|  | 1 x RCA/ composite: VCR |  |
|  | 1 x DVI-I: PC |  |
|  | Video outputs (4 outputs) |  |
|  | 1 x S-video: main monitor |  |
|  | 1 x RCA/composite: main monitor or VCR |  |
|  | 1 x RCA/composite: dual monitor or VCR |  |
|  | 1 x DVI/XGA: main or second monitor |  |
|  | Video resolution |  |
|  | XGA (1024x768), SVGA (800x600), SXGA (1280x1024) , HD 1280x720 |  |
|  | 720p, 30fps at 1Mbps–4Mbps |  |
|  | 4SIF (704 x 480)/4CIF (704 x 576) at 256Kbps–1Mbps |  |
|  | SIF (352 x 240), CIF (352 x 288) |  |
|  | QSIF (176 x 120), QCIF (176 x 144) |  |
|  | Audio features |  |
|  | Stereo |  |
|  | 22kHz high fidelity |  |
|  | Automatic gain control |  |
|  | Automatic noise suppression |  |
|  | Audio error concealment |  |
|  | Echo cancellation and far-end feedback reduction device |  |
|  | Fully duplex |  |
|  | Audio inputs |  |
|  | 2 x microphone |  |
|  | 1 x RCA |  |
|  | 1 x RCA |  |
|  | Audio outputs |  |
|  | 1 x RCA: main audio |  |
|  | 1 x RCA: VCR or Analogue Stereo R |  |
|  | Network |  |
|  | Interfaces: - 2 10/100 auto NIC, RJ45 connectors - RJ11 analog phone connector |  |
|  | H.323 up to 4 Mbps point-to-point |  |
|  | SIP up to 4 Mbps |  |
|  | USB 2.0 connector |  |
|  | Intelligent packet loss recovery |  |
|  | Dynamic Bandwidth Allocation (DBA) |  |
|  | Reconfigurable MTU size |  |
|  | User interface |  |
|  | Remote controller for all devices |  |
|  | System management via embedded web server, SNMP, Telnet, SSH, FTP |  |
|  | Directory services |  |
|  | Remote software upload: via web server, ftp server or ISDN |  |
|  | 1 x RS-232 local control and diagnostics |  |
|  | On-screen menu system |  |
|  | Security features |  |
|  | IP Administration Password |  |
|  | Menu Administration Password |  |
|  | Dialling Access code |  |
|  | Streaming password |  |
|  | VNC password |  |
|  | SNMP security alerts |  |
|  | Disable IP services |  |
|  | Network Settings protection |  |
|  | Secure password authentication |  |
|  | Camera |  |
|  | 2 (two) High definition pan/tilt/zoom cameras |  |
|  | Resolution: 1280x720, 30 fps |  |
|  | Automatic or manual focus |  |
|  | 12 x zoom +20°/-30° tilt +/- 180° pan |  |
|  | 72deg FOV at min zoom |  |
|  | Far end camera control |  |
|  | 2 year warranty (parts and labour) |  |
|  | Document camera |  |
|  | XGA resolution (1024 x 768) |  |
|  | 850,000 pixel CCD |  |
|  | 10x zoom |  |
|  | Repositionable top lights |  |
|  | Wireless remote control and front panel operation |  |
|  | One-touch freeze function and on-screen display |  |
|  | Input/output: Analog RGB, RS-232C, USB, composite video |  |
|  | Cabling and accessories |  |
|  | Compact lightweight design |  |
|  | 2 year warranty (parts and labour) |  |
|  | Microphone system |  |
|  | 10 (ten) tabletop wireless microphones: Omni-directional (360 degree coverage), rechargeable Lithium Polmer battery (8 hours of talk time), Small form factor, mute button, not affected by RF frequency |  |
|  | Rack mount base unit: seamless connection to video conferencing codec, channel LED indicators, individual and master mute, Audio in and out connections (8 channels in, 8 channels out), local/remote selector switch |  |
|  | Charger base: DC power input port, 8 microphone charge jacks |  |
|  | Plug and Play |  |
|  | Two-way audio |  |
|  | 1Year Warranty (parts and labour) |  |
|  | Speakers |  |
|  | Front-firing stereo speaker kit (two speakers with stands and subwoofer) |  |
|  | 2 directional ceiling speakers |  |
|  | Cabling (wireless receiver) and installation accessories |  |
|  | Display devices |  |
|  | Plasma screen (to be mounted on wall) |  |
|  | 42" screen, 1366 x 768 HDTV Resolution |  |
|  | 16:9 Widescreen HDTV format |  |
|  | 10000:1 Contrast Ratio |  |
|  | Image Sticking Minimization: Pixel Orbiter, White Wash, and Inversion Features |  |
|  | 16:9, 4:3 Aspect Ratio Correction |  |
|  | Inbuilt speakers with Surround |  |
|  | Inputs:1 Composite Video, 1 S-Video, 1 Component Video, 1 VGA, 1 DVI, 2 Stereo Audio In, 1Serial Control |  |
|  | Outputs: 1 VGA and 1 Stereo Audio |  |
|  | Picture-Out-of-Picture (POP) |  |
|  | Picture-in-Picture |  |
|  | Twin Picture |  |
|  | Intelligent Image Scaling |  |
|  | 2 Year Warranty (parts and labour) |  |
|  | Interactive whiteboard (77") |  |
|  | Notebook software |  |
|  | Pen Tray (with optical sensors) and 4 pens (black, blue, red and green) and en eraser |  |
|  | 4000 × 4000 touch resolution |  |
|  | Hard-coated polyester screen (compatible with dry-erase markers and cleans easily with whiteboard cleaner) |  |
|  | Floor stand |  |
|  | Two year warranty (parts and labour) |  |
|  | Architectural, civil works and furnishing |  |
|  | Lighting (Special lights for Video conferencing) |  |
|  | Heavy acoustic quality curtains (blue/grey in colour). Sample to be presented before installation. |  |
|  | Trunking (Marshall Tufflex 100 x 50mm, with dividing fillet to separate power cables from other cables) |  |
|  | Data installation |  |
|  | Cisco switch |  |
|  | Category 6 unshielded twisted pair cable |  |
|  | 6-core fiber (multimode) |  |
|  | Data Outlets:Doucle white coloured, shuttered and angled |  |
|  | Cable managers |  |
|  | Fiber patch panel |  |
|  | Copper patch panel |  |
|  | Copper patch cords (both 3m and 1m) |  |
|  | Fiber patch cords |  |
|  | Cisco Catalyst CE500G-12TC |  |
|  | 2 SFP moudles (GLC-SX-MM=) |  |
|  | Power installation |  |
|  | Dedicated Main Distribution Board |  |
|  | Rack mount UPS |  |
|  | Power cable (mains power) |  |
|  | Power cable (for sockets) |  |
|  | Power outlets |  |
|  | Partition for control room |  |
|  | Aluminium frame |  |
|  | Block board filling |  |
|  | Testing of installation |  |
|  | The entire installation tests shall be tested in the presence of the end users to demonstrate that all functionality is as designed and intended. |  |
|  | Documentation |  |
|  | Upon successful completion of installation and testing, the supplier shall provide "As-installed" drawings together with complete operation and maintenance instruction manuals. |  |
|  | Warranty |  |
|  | All components supplied and installed under this contract shall be warranted against defects in design, materials and workmanship For items whose warranty has not been explicitly stated, warranty (parts and labour) should be provided for one year. |  |
|  | Training |  |
|  | Prior to final system acceptance, the bidder shall provide on-site training for 4 people in the function and use of the system. |  |

The detailed technical evaluation will examine the technical specification of the items offered in column c and determine whether this meets the minimum specification in column b.  Bidders must complete column c or the bid will be rejected.  Bidders are required to include technical literature to support the details provided in column c.

- Drawings

Procurement Reference Number: MUK/SPLS/2007-08/00121

| List of related Drawings |  |  |
| --- | --- | --- |
| Drawing number | Drawing name | Purpose |
| 1 | Video conferencing room layout | To show where each component will placed |

- Inspections and Tests

Procurement Reference Number: MUK/SPLS/2007-08/00121

| List of Inspections and Tests |  |
| --- | --- |
| Items subject to Inspection and Tests; | One |
| Type of inspection or tests and the standards to be met; | - Physical inspection of the specifications - Equipment checkout - Acceptance testing |
| Location of the inspection or tests; | - Faculty of Economics and Management |
| Inspection agency; | - Faculty of Economics and Management - Directorate for ICT Support |
| Timing of the inspection; | Before, during and after installation |
| Notifications or documentation required from the provider; | - Copy of manual and operating guide - Details of the Bidder’s Quality Assurance procedures - Warranty documents - After sales support |
| Provision of any samples for inspection; |  |
| Cost of the inspection; |  |
| Arrangements and costs for any re-inspection required; |  |
| Any other relevant details. |  |